NETWORK INFRASTRUCTURE

Knowledge Base — Standard Operating Procedures

Tata Consultancy Services

Network Infrastructure & Security Division

| Document ID | Version | Date | Classification |
| --- | --- | --- | --- |
| TCS-NET-KB-002 | v1.0 | June 2025 | Internal Confidential |

Sections Covered

1. F5 BIG-IP Firmware Upgrade

2. F5 SSL Certificate Renewal

3. Cisco Nexus Switch Firmware Upgrade

4. F5 BIG-IP Failover Testing

5. Switch VLAN Configuration

6. Switch Trunk Configuration

7. BGP Troubleshooting

# 1. F5 BIG-IP Firmware Upgrade Process

This SOP covers the end-to-end process for upgrading F5 BIG-IP software in a high-availability (HA) Active/Standby pair. Always upgrade the Standby unit first.

## 1.1 Pre-Upgrade Checklist

| Check Item | Command / Action | Expected Result |
| --- | --- | --- |
| Verify HA Pair Status | show sys failover | Standby unit shows 'standby' |
| Check active connections | show sys connection count | Document baseline count |
| Verify disk space | show sys disk | At least 4 GB free on /var |
| Backup UCS configuration | tmsh save sys ucs /var/local/ucs/pre_upgrade.ucs | File created successfully |
| Download firmware ISO | Download from downloads.f5.com to /shared/images/ | MD5 checksum verified |
| Check current version | show sys version | Document current version |
| Notify stakeholders | Send maintenance notification email | Confirmation received |

⚠  CRITICAL: Always upgrade Standby first. Never upgrade Active unit without confirming Standby is healthy post-upgrade.

## 1.2 Upgrade Steps — Standby Unit

- SSH into the Standby BIG-IP unit.

- Upload firmware ISO: scp <iso-file> root@<standby-ip>:/shared/images/

- Verify MD5 checksum: md5sum /shared/images/<iso-file>

- Install software to inactive boot location: tmsh install sys software image <iso-file> volume HD1.2

- Monitor installation progress: watch -n 5 tmsh show sys software

- Reboot to new volume: tmsh reboot volume HD1.2

- Wait 5–7 minutes for unit to come back online.

- Verify new version: tmsh show sys version

- Verify HA status: tmsh show sys failover — unit should show 'standby'.

- Run health checks: tmsh show ltm virtual — all virtuals should show 'available'.

## 1.3 Upgrade Steps — Active Unit (Failover First)

- Initiate controlled failover from Active to Standby: tmsh run sys failover standby

- Confirm original Standby is now Active: tmsh show sys failover

- Verify traffic is flowing on new Active unit.

- Notify team that failover is complete before proceeding.

- Repeat steps 1.2 (1–10) on the original Active unit (now Standby).

- After upgrade, fail back to original Active if required: tmsh run sys failover standby

## 1.4 Post-Upgrade Verification

| Verification Step | Command | Pass Criteria |
| --- | --- | --- |
| Software version | tmsh show sys version | New version displayed |
| HA sync status | tmsh show cm sync-status | In Sync |
| Virtual server status | tmsh show ltm virtual | All 'available (enabled)' |
| Pool member status | tmsh show ltm pool | All members 'up' |
| SSL certificate status | tmsh show ltm ssl-cert | No expired certs |
| Traffic baseline | show sys connection count | Within 10% of pre-upgrade |

## 1.5 Rollback Procedure

⚠  If upgrade fails or any virtual server is down after upgrade, execute rollback immediately.

- Boot back to previous volume: tmsh reboot volume HD1.1

- Restore UCS backup if config is corrupt: tmsh load sys ucs /var/local/ucs/pre_upgrade.ucs

- Verify all services are restored.

- Raise incident ticket and contact F5 TAC: 1-888-882-7535

# 2. F5 SSL Certificate Renewal Process

This SOP covers SSL/TLS certificate renewal for F5 BIG-IP virtual servers. Follow this process 30 days before certificate expiry.

## 2.1 Certificate Expiry Check

- List all certificates and expiry dates: tmsh list sys file ssl-cert | grep -E 'name|expiration'

- Identify certificates expiring within 30 days.

- Note the CN (Common Name) and SAN (Subject Alternative Names) from the existing cert.

- Raise change request for certificate renewal.

## 2.2 Generate New CSR on F5

- Create new private key: tmsh create sys crypto key <key-name> key-size 2048 security-type normal

- Generate CSR: tmsh create sys crypto csr <csr-name> common-name <FQDN> key <key-name> country IN state WB city Kolkata organization TCS

- Export CSR: tmsh list sys crypto csr <csr-name>

- Copy CSR content and send to Certificate Authority (CA) for signing.

## 2.3 Install Renewed Certificate

- Receive signed certificate file (.crt) from CA.

- Upload certificate to F5: tmsh install sys crypto cert <cert-name> from-local-file /var/tmp/<cert-file>.crt

- Upload intermediate/chain cert if provided: tmsh install sys crypto cert <chain-name> from-local-file /var/tmp/chain.crt

- Update SSL profile to use new cert: tmsh modify ltm profile client-ssl <profile-name> cert <cert-name> key <key-name> chain <chain-name>

- Save configuration: tmsh save sys config

## 2.4 Certificate Renewal Verification

| Verification | Method | Pass Criteria |
| --- | --- | --- |
| Certificate installed | tmsh list sys file ssl-cert <cert-name> | New expiry date shown |
| SSL profile updated | tmsh list ltm profile client-ssl <profile-name> | New cert/key referenced |
| Browser test | Open HTTPS URL in browser | Padlock shows new cert, no warnings |
| SSL Labs test | ssllabs.com/ssltest (for internet-facing) | Grade A or above |
| No expired certs | tmsh show ltm ssl-cert | No 'expired' status |

📝  NOTE: Certificate changes replicate automatically to the Standby unit via config sync. Verify sync status after saving.

# 3. Cisco Nexus Switch Firmware Upgrade

This SOP covers NX-OS firmware upgrade for Cisco Nexus 9000/7000/5000 series switches. Nexus supports In-Service Software Upgrade (ISSU) on supported platforms to minimize downtime.

## 3.1 Pre-Upgrade Checks

| Check | Command | Expected |
| --- | --- | --- |
| Current NX-OS version | show version | Document baseline |
| Check ISSU compatibility | show incompatibility-all nxos <image> | No incompatibilities |
| VPC peer status | show vpc | vPC peer: peer is alive |
| BGP/OSPF neighbors | show bgp summary / show ip ospf neighbor | All neighbors up |
| Spanning tree state | show spanning-tree summary | No TCN in progress |
| Available space | show system internal flash | Min 1.5 GB free |
| Backup running config | copy running-config bootflash:pre_upgrade_config.txt | Copy complete |

⚠  ISSU is supported only on Nexus 7000 and 9000 with N9K-SUP-A+ supervisor. Nexus 5000/3000 require a maintenance window with downtime.

## 3.2 Copy Firmware Image

- Copy new NX-OS image to bootflash: copy scp://<server>/<image.bin> bootflash: vrf management

- Verify MD5 hash: show file bootflash:<image.bin> md5sum

- Compare with hash published on Cisco CCO download page.

## 3.3 ISSU Upgrade (Non-Disruptive — Nexus 9K/7K)

- Check ISSU compatibility: show incompatibility-all nxos bootflash:<image.bin>

- Run ISSU install: install all nxos bootflash:<image.bin>

- Accept the prompt to proceed when compatibility check passes.

- Monitor upgrade progress — takes 20–40 minutes: show install all status

- Switch will reload modules sequentially with no traffic loss.

- After completion, verify: show version

## 3.4 Standard Upgrade (Disruptive — Nexus 5K/3K)

- Schedule maintenance window (minimum 30 minutes downtime).

- Set boot variable: boot nxos bootflash:<image.bin>

- Save config: copy running-config startup-config

- Reload switch: reload

- After reload, verify version: show version

- Verify all interfaces are up: show interface status

- Verify BGP/OSPF neighbors are re-established.

## 3.5 Post-Upgrade Verification

| Check | Command | Pass Criteria |
| --- | --- | --- |
| NX-OS version | show version | New version confirmed |
| VPC status | show vpc | Peer alive, vPC up |
| All interfaces up | show interface status ¦ inc connected | All expected ports connected |
| BGP neighbors | show bgp summary | All peers Established |
| Spanning tree | show spanning-tree summary | No topology changes |
| Logs clean | show logging last 50 | No critical errors |

# 4. F5 BIG-IP Failover Testing SOP

Failover testing should be performed quarterly during a maintenance window to validate HA functionality. Testing confirms that traffic shifts seamlessly from Active to Standby unit.

## 4.1 Pre-Test Checklist

| Item | Action | Verified By |
| --- | --- | --- |
| Maintenance window | Approved RFC in place | Change Manager |
| Stakeholder notification | Email sent to all teams 48 hrs prior | Team Lead |
| Baseline traffic | Document active connections and VIP response times | Engineer |
| HA sync status | tmsh show cm sync-status — must show 'In Sync' | Engineer |
| Monitoring | Ensure SolarWinds/Nagios alerts are suppressed for window | NOC |

## 4.2 Failover Test Steps

- Document Active unit hostname: tmsh show sys failover

- Record baseline: tmsh show sys connection count

- Test VIP response: curl -k https://<VIP-IP> — note response time.

- Initiate failover: tmsh run sys failover standby

- Immediately monitor traffic cutover — should complete in under 3 seconds.

- Confirm new Active unit: tmsh show sys failover on both units.

- Test VIP again: curl -k https://<VIP-IP> — traffic must continue without error.

- Check connection count on new Active: tmsh show sys connection count

- Hold for 10 minutes — monitor for any anomalies.

- Fail back to original Active: tmsh run sys failover standby on current Active.

- Verify config sync: tmsh show cm sync-status — must show 'In Sync'.

## 4.3 Failover Test Results Template

| Test Parameter | Pre-Failover | Post-Failover | Pass / Fail |
| --- | --- | --- | --- |
| Active Unit | Unit-A | Unit-B |  |
| Failover Time | N/A | < 3 seconds |  |
| VIP HTTP Response | 200 OK | 200 OK |  |
| Active Connections | [Baseline] | Within 5% of baseline |  |
| HA Sync Status | In Sync | In Sync |  |
| Config Version | [Ver#] | Same version |  |

⚠  If failover time exceeds 10 seconds or VIP is unreachable after failover, do NOT fail back. Raise P2 incident immediately.

# 5. Switch VLAN Configuration SOP

This SOP applies to Cisco Catalyst (IOS/IOS-XE) and Nexus (NX-OS) switches. Follow the change management process before adding VLANs in production.

## 5.1 VLAN Naming Convention

| VLAN Range | Purpose | Naming Format |
| --- | --- | --- |
| 1 | Default (never use) | Do not assign |
| 2–99 | Management VLANs | MGMT-<location> |
| 100–199 | Server VLANs | SRV-<app-name> |
| 200–299 | User / Desktop VLANs | USR-<floor/dept> |
| 300–399 | Voice VLANs | VOICE-<location> |
| 400–499 | DMZ / Security VLANs | DMZ-<zone> |
| 900–999 | Storage VLANs | SAN-<zone> |

## 5.2 Create VLAN — Cisco IOS/IOS-XE

- Enter global config: conf t

- Create VLAN and assign name: vlan <ID>  /  name <VLAN-NAME>

- Exit VLAN config: exit

- Assign VLAN to access port:

- interface GigabitEthernet1/0/X

- switchport mode access

- switchport access vlan <ID>

- spanning-tree portfast

- no shutdown

- Verify VLAN created: show vlan brief

- Verify port assignment: show interfaces GigabitEthernet1/0/X switchport

- Save configuration: write memory

## 5.3 Create VLAN — Cisco Nexus NX-OS

- Enter config mode: config t

- Create VLAN: vlan <ID>  /  name <VLAN-NAME>  /  state active  /  no shutdown

- Assign to interface: interface Ethernet1/X  /  switchport  /  switchport mode access  /  switchport access vlan <ID>

- Verify: show vlan brief  and  show interface Ethernet1/X switchport

- Save: copy running-config startup-config

## 5.4 VLAN Configuration Verification

| Verification | IOS Command | NX-OS Command |
| --- | --- | --- |
| List all VLANs | show vlan brief | show vlan brief |
| Port VLAN assignment | show interfaces <int> switchport | show interface <int> switchport |
| MAC address table | show mac address-table vlan <ID> | show mac address-table vlan <ID> |
| STP per VLAN | show spanning-tree vlan <ID> | show spanning-tree vlan <ID> |

# 6. Switch Trunk Configuration SOP

Trunk ports carry multiple VLANs between switches, between switches and routers, or between switches and servers (for VLAN tagging). IEEE 802.1Q is the standard used.

## 6.1 Trunk Port Design Rules

- Always use 802.1Q encapsulation (not ISL — ISL is deprecated).

- Define an explicit native VLAN — never use VLAN 1 as native VLAN in production.

- Always use 'switchport trunk allowed vlan' to restrict to only required VLANs.

- Enable BPDU Guard on access ports — never on trunk ports.

- Disable DTP (Dynamic Trunking Protocol) by setting mode explicitly.

## 6.2 Configure Trunk — Cisco IOS/IOS-XE

- Enter interface config: interface GigabitEthernet1/0/X

- Set trunk mode: switchport mode trunk

- Set 802.1Q encapsulation (if required): switchport trunk encapsulation dot1q

- Set native VLAN (use dedicated native VLAN, not VLAN 1): switchport trunk native vlan 999

- Restrict allowed VLANs: switchport trunk allowed vlan 10,20,30,100,200

- Disable DTP: switchport nonegotiate

- Enable port: no shutdown

- Save config: write memory

## 6.3 Configure Trunk — Cisco Nexus NX-OS

- Enter interface: interface Ethernet1/X

- Set switchport mode: switchport  /  switchport mode trunk

- Set native VLAN: switchport trunk native vlan 999

- Allow specific VLANs: switchport trunk allowed vlan 10,20,30,100,200

- No shutdown: no shutdown

- Save: copy running-config startup-config

## 6.4 Trunk Verification Commands

| Verification | IOS Command | NX-OS Command |
| --- | --- | --- |
| Trunk status | show interfaces trunk | show interface trunk |
| Allowed VLANs | show interfaces <int> trunk | show interface <int> trunk |
| Native VLAN | show interfaces <int> switchport | show interface <int> switchport |
| VLANs forwarding | show interfaces trunk (VLANs in STP) | show spanning-tree vlan <ID> |
| CDP neighbor | show cdp neighbors <int> detail | show cdp neighbors <int> detail |

## 6.5 Common Trunk Issues & Fixes

| Issue | Cause | Fix |
| --- | --- | --- |
| VLAN not passing traffic | VLAN not in allowed list | Add VLAN: switchport trunk allowed vlan add <ID> |
| Native VLAN mismatch | Both sides have different native VLAN | Match native VLAN on both ends |
| Port in err-disabled | BPDU guard triggered on trunk port | Check config — ensure BPDU guard is off on trunk |
| DTP negotiation loop | DTP enabled on both sides | Set 'switchport nonegotiate' on both ends |

# 7. BGP Troubleshooting SOP

This SOP provides a structured approach to troubleshoot BGP issues on Cisco IOS, IOS-XR, and Nexus platforms. BGP problems typically fall into three categories: session not forming, routes not received, or suboptimal routing.

## 7.1 BGP Troubleshooting Decision Tree

| Symptom | First Command | Common Cause |
| --- | --- | --- |
| BGP session is Idle | show bgp neighbors <IP> | TCP connectivity issue or misconfigured neighbor IP |
| BGP session is Active | telnet <neighbor-IP> 179 | Firewall blocking TCP 179 or wrong update-source |
| BGP session is OpenSent | show bgp neighbors ¦ inc hold | Hold timer mismatch or AS number mismatch |
| BGP session is Established but no routes | show bgp neighbors <IP> received-routes | Route policy filtering all prefixes |
| Routes received but not in routing table | show bgp <prefix> | Local preference or weight issue — another path preferred |
| Routes in table but traffic blackholed | show ip cef <prefix> | Recursive lookup failure — next-hop unreachable |
| BGP session flapping | show interfaces <WAN-int> | Unstable WAN link or MTU mismatch |

## 7.2 BGP Session Not Forming — Checklist

- Verify neighbor IP and AS number in config: show run | sec router bgp

- Ping neighbor IP from correct source: ping <neighbor-IP> source <loopback/interface>

- Test TCP port 179 reachability: telnet <neighbor-IP> 179

- Check ACL/firewall: ensure TCP 179 is permitted bidirectionally.

- Verify update-source matches loopback (for iBGP): show bgp neighbors <IP> | inc Local host

- Check BGP authentication: if MD5 is used, verify passwords match on both sides.

- Review BGP logs: debug ip bgp <neighbor-IP> events (use with caution on production).

## 7.3 BGP Routes Not Received

- Check if routes are being sent by neighbor: show bgp neighbors <IP> received-routes

- If command shows nothing, run: neighbor <IP> soft-reconfiguration inbound (then retry)

- Check inbound route policy: show route-policy <inbound-policy-name>

- Verify prefix-list is not filtering: show ip prefix-list <name>

- Check maximum-prefix limit: show bgp neighbors <IP> | inc Maximum

- Verify neighbor is in correct address-family: show bgp all summary

## 7.4 BGP Best Path Not Selected Correctly

BGP path selection order (remember: We Love Oranges As Oranges Mean Pure Refreshment):

| Step | Attribute | Prefer Higher/Lower | Admin Control |
| --- | --- | --- | --- |
| 1 | Weight | Higher | Cisco proprietary, local only |
| 2 | Local Preference | Higher | set local-preference in route-policy |
| 3 | Locally Originated | Prefer local | network/redistribute vs received |
| 4 | AS Path Length | Shorter | prepend AS path to make route less preferred |
| 5 | Origin Code | IGP > EGP > ? | set origin in route-policy |
| 6 | MED | Lower | set med in route-policy |
| 7 | eBGP over iBGP | eBGP preferred | topology dependent |
| 8 | IGP metric to next-hop | Lower | optimize IGP |

- Check best path details: show bgp <prefix> detail

- Compare attributes of competing paths using above table.

- Adjust local-preference for inbound preference: route-policy IN / set local-preference 200 / end-policy

- Adjust AS-path prepend for outbound preference: route-policy OUT / prepend as-path <ASN> 3 / end-policy

## 7.5 BGP Session Flapping — Root Cause Steps

- Check interface error counters on WAN link: show interfaces <WAN-int> | inc error|reset

- Check BGP keepalive and hold timers: show bgp neighbors <IP> | inc hold|keepalive

- Check MTU — BGP UPDATE messages can be large: show interfaces <int> | inc MTU

- Test with extended ping using 1500-byte packets: ping <neighbor-IP> size 1500 df-bit

- If MTU mismatch found: set MTU consistently on both ends (ip mtu 1500 on interface).

- Check for CPU spikes during flap: show processes cpu sorted | head 15

- Review BGP notifications sent/received: show bgp neighbors <IP> | inc notifications

## 7.6 Useful BGP Commands — Quick Reference

| Command | Platform | Purpose |
| --- | --- | --- |
| show bgp summary | IOS/IOS-XR/NX-OS | Overview of all BGP peers and prefix counts |
| show bgp neighbors <IP> | IOS/IOS-XR | Detailed BGP session info, timers, capabilities |
| show bgp neighbors <IP> received-routes | IOS | All routes received from peer (requires soft-reconfig) |
| show bgp neighbors <IP> advertised-routes | IOS | All routes advertised to peer |
| show bgp <prefix> | IOS/NX-OS | Best path and all paths for specific prefix |
| show route-policy <name> | IOS-XR | Display route policy logic |
| clear bgp <IP> soft in | IOS | Soft reset inbound — no session drop |
| clear bgp <IP> soft out | IOS | Soft reset outbound — re-advertise routes |
| debug ip bgp <IP> events | IOS | BGP event debug — use with caution on production |
| show bgp process | NX-OS | BGP process-level details and memory |

📝  NOTE: Always use 'soft' clear commands in production. Hard reset (clear bgp <IP>) drops the TCP session and causes traffic disruption.

— End of Knowledge Base Document —